CLASSIFICATION OF BUILDINGS BASED ON OCCUPANCY

Reference: NBC India 2016-Part 4, Clause 3.1

    - General Classification

All buildings, whether existing or hereafter erected shall be classified according to use or the character of occupancy in one of the following groups:

Group A Residential

| Sub-division A-1 | Lodging and rooming houses, and dormitories |
| --- | --- |
| Sub-division A-2 | One or two family private dwellings |
| Sub-division A-3 | Apartment houses |
| Sub-division A-4 | Hotels |

Group B Educational

| Sub-division B-1 | Schools up to senior secondary level |
| --- | --- |
| Sub-division B-2 | All others/training institutions , a group of buildings under single management which is used for students not less than 100 in number |

Group C Institutional

| Sub-division C-1 | Hospitals and sanatoria |
| --- | --- |
| Sub-division C-2 | Custodial institutions |
| Sub-division C-3 | Penal and mental institutions |

Group D Assembly

| Sub-division D-1 | Buildings having a theatrical or motion picture or any other stage and fixed seats for over 1 000 persons |
| --- | --- |
| Sub-division D-2 | Buildings having a theatrical or motion picture or any other stage and fixed seats up to 1 000 persons |
| Sub-division D-3 | Buildings without a permanent stage having accommodation for 300 or more persons but no permanent seating arrangement |
| Sub-division D-4 | Buildings without a permanent stage having accommodation for less than 300 persons with no permanent seating arrangement |
| Sub-division D-5 | All other structures including temporary structures designed for assembly of people not covered by subdivisions D-1 to D-4, at ground level. |
| Sub-division D-6: | Buildings having mixed occupancies such as shopping malls (providing facilities such as shopping, cinema theatres, multiplexes and restaurants/food courts) and having other occupancies such as offices, retail, residential mercantile etc., coexisting in combination of two or more occupancies. |
| Sub-division D-7 | Underground and elevated mass rapid transit system |

Group E Business

| Sub-division E-1 | Offices, banks, professional establishments, like offices of architects, engineers, doctors, lawyers, post offices and police stations |
| --- | --- |
| Sub-division E-2 | Laboratories, outpatient clinics, research establishments, libraries and test houses. |
| Sub-division E-3 | Electronic data processing centers, computer installations, Information technology parks and call centers |
| Sub-division E-4 | Telephone exchanges. |
| Sub-division E-5 | Broadcasting stations, T.V. Stations and air traffic control towers |

Group F Mercantile

| Sub-division F-1 | Shops, stores, departmental stores, markets (any with covered area up to 500 m2). |
| --- | --- |
| Sub-division F-2 | Shops, stores, departmental stores, markets (any with covered area more than 500 m2). |
| Sub-division F-3 | Underground shopping centers |

Group G Industrial

| Sub-division G-1 | Buildings used for low hazard industries. |
| --- | --- |
| Sub-division G-2 | Buildings used for moderate hazard industries. |
| Sub-division G-3 | Buildings used for high hazard industries |

Group H Storage

These shall include any building or part of a building used primarily for the storage or sheltering (including servicing, processing or repairs incidental to storage) of goods, ware or merchandise (except those that involve highly combustible or explosive products or materials) vehicles or animals,

Group J Hazardous

These shall include any building or part thereof which is used for the storage, handling, manufacture or processing of highly combustible or explosive materials or products which are liable to burn with extreme rapidity and/or which may produce poisonous fumes or explosions for storage, handling, manufacturing or processing which involve highly corrosive, toxic or noxious alkalis, acids or other liquids or chemicals producing flame, fumes and explosive, poisonous, irritant or corrosive gases; and for the storage, handling or processing of any material producing explosive mixtures of dust which result in the division of matter into fine particles subject to spontaneous ignition

Hazard Level classification:

Referance: NBC india 2016, ANNEX B

| Light Hazard Occupancies | Moderate Hazard Occupancies | High Hazard Occupancies |
| --- | --- | --- |
| Glass & Glass Fibre Manufacturing Godowns and Warehouses Storing non- combustible Goods only Green houses Gold Thread /Gilding Factories Gum and/or Glue and Gelatine Manufacturing Ice, Ice Candy and Ice-cream Manufacturing Ink (excluding Printing Ink) Factories Mica Products Manufacturing Multiple block apartment buildings Multiple block business buildings Multiple block star hotel buildings Places of worship Pottery Works Poultry Farms Salt Crushing Factories and Refineries Stables Sugar Candy Manufacturing Sugar Factories and Refineries Tanneries/Leather Goods Manufacturers Umbrella Assembling Factories Vermicelli Factories Water Treatment/ Filtration Plants and Water Pump Houses Zinc/Copper Factories | Ghee Factories (other than vegetable) Godowns and Warehouses (other than those under light and High Hzard A categories) Grain and/or Seeds Disintegrating and/or Crushing Factories Grease Manufacturing. Hosiery, Lace, Embroidery and Thread Factories Incandescent Gas Mantle Manufacturing Industrial Gas Mfg. (Inert/ halogenated hydrocarbon gases) Man-made Yarn/Fibre Manufacturing (other than Acrylic fibres/yarn mfg) Manure and Fertiliser Works. (Blending, Mixing and granulating) Mineral Oil Blending and Processing Museums Oil and Leather Cloth Factories. Oil Terminals/Depots other than those categorised under High hazard A Open storage of flammable liquids in drums, cans etc Oxygen Plants Paper and Cardboard Mills. without Raw Material Yards Piers, Wharves jetties and Dockyards other than those categorized under High hazard A Plastic Goods Manufacturing Plywood /Wood Veneering Factories Printing Press Premises Pulverising and Crushing Mills Rice Mills Rope Works Rubber Goods Manufacturing Rubber Tyres and Tubes Manufacturing Shellac Factories Silk Filatures Soaps and Glycerine Factories. Spray painting Starch Factories Tea Factories Textile Mills Tobacco (Chewing) and Pan-Masala Making Tobacco Grinding and Crushing Tobacco Redrying Factories Woollen Mills | Oil Extraction Plants Oil Terminals/Depots handling flammable liquids having flash point of 23o C and below. Paints & Varnish Factories Paper and cardboard mills having raw material yards Piers, Wharfs and Jetties – handling extra hazardous materials Printing Ink Manufacturing. Rosin Lamp black and Turpentine Factories Saw Mills Sponge Iron Steel Plants (Gas Based) Surgical Cotton Manufacturing Tarpaulin and Canvas Proofing Factories Turpentine and Rosin Distilleries Tyre Retreading and Resoling Factories SUB-CATEGORY (B) Ammonia and Urea Synthesis Plants CNG Compressing and Bottling Plants Coal based methane plants Explosive Factories NOTE — In case of complexes having separate plants having varying degrees of hazard, authority having jurisdiction shall be consulted to decide on level of protection to be provided. |

Pipe diameter and the maximum number of sprinkler connections allowed, based on pipe size and material.

Referance: NFPA-13, Table 28.5.3.4 Ordinary Hazard pipe schedule